PART 1 — ENGLISH

# Request for Budgetary Technical and Commercial Proposal No. OVE75-RFQ-07

Subject: Drum vacuum filters for copper concentrate slurry.

KVANT LLC (Russian Federation), within the Basic Engineering (FEED) stage of the construction project of a copper production plant — a roasting–leaching–electrowinning complex with a capacity of 75,000 tonnes of copper cathodes per year — kindly requests your budgetary technical and commercial proposal (budgetary quotation) for the item above. Project site: an operating metallurgical site in north-west Russia.

Brief description of the item: Cake moisture after filtration 10 %; complete with the vacuum system, filtrate removal and cake discharge into a screw mixer. Technical requirements are given in Annex 1 to this request.

Please include in your quotation:

- budget price quoted separately on EXW (manufacturer’s works) and DDP project site in north-west Russia terms (Incoterms 2020), stating the currency and the validity period of the offer;

- manufacturing lead time and estimated delivery time (weeks from order / advance payment);

- overall dimensions and weights of the equipment and of the largest shipping blocks (for logistics studies);

- reference list of similar supplies: product, process duty and parameters, year of supply, country;

- materials of parts in contact with the process medium, and the scope of supply (instrumentation, spare parts, documentation);

- utility consumption and installation requirements (if available).

This is a budgetary enquiry at the Basic Engineering (FEED) stage: it does not constitute an offer and creates no obligations for either party; detailed technical data sheets will be issued at the next project stage.

Reply deadline: we kindly ask you to submit your quotation within 3 (three) weeks from receipt of this request to stepan@kvantpro.com. Please confirm receipt of this request.

Confidentiality. This request and the attached technical materials are provided solely for the preparation of the quotation, are confidential, and shall not be disclosed to third parties, copied or published without the prior written consent of KVANT LLC. Submission of a quotation creates no legal obligations for either party.

Contact person: Stepan Kharkov, KVANT LLC, stepan@kvantpro.com.

Annex 1: Technical requirements.

# Annex 1. Technical requirements

To RFQ No. OVE75-RFQ-07 — values follow the Customer’s input data and terms of reference; Basic Engineering (FEED) stage, subject to refinement at the next project stage.

| RFQ item | Drum vacuum filters for copper concentrate slurry |
| --- | --- |
| Summary | Cake moisture after filtration 10 %; vacuum system, filtrate removal and cake discharge into a screw mixer |
| Specific requirements | Guaranteed cake moisture of 10 % on concentrate with a particle size of 25.5 µm at 70 % solids in the feed |

Equipment covered by this request:

## Drum vacuum filters for copper concentrate slurry   Concentrate filtration

| Duty | Dewatering of the thickened copper concentrate slurry from the matte separation section (cake moisture 10 %), with the vacuum system, filtrate removal and cake discharge into a screw mixer |
| --- | --- |
| Key parameters | Per the Basic Engineering terms of reference, clause 3.4 |
| Quantity | To be confirmed during basic engineering |

## Filtrate receiving tank downstream of the vacuum filter, with pumping equipment   Concentrate filtration

| Duty | Receiving and pumping out of the filtrate |
| --- | --- |
| Quantity | Complete set |

The quotation must confirm: process medium, temperature, pressure, materials of wetted parts, capacity, dimensions and weight, power consumption, manufacturing lead time, delivery terms.

PART 2 — 中文 (CHINESE)

# 预算性技术商务报价询价函 编号 OVE75-RFQ-07

事由：铜精矿矿浆转鼓真空过滤机。

KVANT LLC（俄罗斯联邦）正在开展铜冶炼厂建设项目的基础设计（FEED）——“焙烧—浸出—电积”联合工艺，年产阴极铜 75,000 吨——现请贵司就上述设备提交预算性技术商务报价（预算报价）。项目场地：俄罗斯西北部一座在运营的冶金厂区。

设备简要说明：过滤后滤饼水分 10%；配真空系统、滤液排出系统及滤饼卸入螺旋混合机的卸料装置。技术要求见本询价函附件 1。

请在报价中包含：

- 预算价格——分别按 EXW（制造商工厂）及 DDP 至俄罗斯西北部项目场地（Incoterms 2020）条件报价，注明币种及报价有效期；

- 制造周期及预计交货期（自订单/预付款之日起按周计）；

- 设备单机及最大运输单元的外形尺寸和重量（用于物流研究）；

- 类似供货业绩表：产品、工况介质及参数、供货年份、国家；

- 与工艺介质接触部件的材质，以及供货范围（仪表、备件、技术文件）；

- 公用工程消耗及安装要求（如有资料）。

本询价为基础设计（FEED）阶段的预算性询价：不构成要约，双方均不因此承担义务；详细技术数据表将在项目下一阶段发出。

回复期限：请在收到本询价函后 3（三）周内将报价发送至 stepan@kvantpro.com。请确认收到本询价函。

保密条款。本询价函及随附技术资料仅供编制报价使用，属保密信息，未经 KVANT LLC 事先书面同意，不得向第三方披露、复制或发布。提交报价不为双方设立任何法律义务。

联系人：Stepan Kharkov, KVANT LLC, stepan@kvantpro.com。

附件 1：本询价设备的技术要求。

# 附件 1. 技术要求

对应询价函编号 OVE75-RFQ-07 — 参数依据业主原始数据及设计任务书；基础设计（FEED）阶段，后续阶段可能进一步细化。

| 询价设备 | 铜精矿矿浆转鼓真空过滤机 |
| --- | --- |
| 简要说明 | 过滤后滤饼水分 10%；真空系统、滤液排出及滤饼卸入螺旋混合机 |
| 特殊要求 | 在精矿粒度 25.5 µm、给料固含量 70% 条件下保证滤饼水分 10% |

本询价涵盖的设备清单：

## 铜精矿矿浆转鼓真空过滤机   精矿过滤

| 用途 | 高冰镍分离工段浓缩铜精矿矿浆的脱水（滤饼水分 10%），配真空系统、滤液排出及滤饼卸入螺旋混合机 |
| --- | --- |
| 主要参数 | 按基础设计任务书第 3.4 条 |
| 数量 | 待基础设计阶段确定 |

## 真空过滤机后滤液接收槽（含泵送设备）   精矿过滤

| 用途 | 滤液的接收与泵送外排 |
| --- | --- |
| 数量 | 成套 |

报价中须确认：工况介质、温度、压力、接触部件材质、生产能力、外形尺寸及重量、能耗、制造周期、交货条件。